Experiment No.  05

Object: Introduction to VLANs.

- To create VLANs in Switch using CLI mode.

- To assign Ip Address to VLANs in Switch using CLI mode.

- To assign ports of switch to VLANs of Switch using CLI mode.  (single or range)

Date: March 03, 2018

# Configuration Figure:

# Coding:

Switch>en

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#vlan 2

Switch(config-vlan)#name admin

Switch(config-vlan)#ex

Switch(config)#vlan 3

Switch(config-vlan)#name it

Switch(config-vlan)#ex

Switch(config)#vlan 4

Switch(config-vlan)#name acc

Switch(config-vlan)#ex

Switch(config)#ex

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#

Switch#

Switch#

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface range fastEthernet 0/1-2

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 2

Switch(config-if-range)#no shutdown

Switch(config-if-range)#ex

Switch(config)#

Switch(config)#interface range fastEthernet 0/3-4

Switch(config-if-range)#switchport mode access

Switch(config-if-range)#switchport access vlan 3

Switch(config-if-range)#no shutdown

Switch(config-if-range)#ex

Switch(config)#

Switch(config)#interface fastEthernet 0/5

Switch(config-if)#switchport mode access

Switch(config-if)#switchport access vlan 4

Switch(config-if)#no shutdown

Switch(config-if)#ex

Switch(config)#

Switch(config)#ex

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12, Fa0/13

Fa0/14, Fa0/15, Fa0/16, Fa0/17

Fa0/18, Fa0/19, Fa0/20, Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gig0/1

Gig0/2

2    admin                            active    Fa0/1, Fa0/2

3    it                               active    Fa0/3, Fa0/4

4    acc                              active    Fa0/5

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

2    enet  100002     1500  -      -      -        -    -        0      0

3    enet  100003     1500  -      -      -        -    -        0      0

4    enet  100004     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Remote SPAN VLANs

------------------------------------------------------------------------------

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

Switch#

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface vlan 2

Switch(config-if)#

%LINK-5-CHANGED: Interface Vlan2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan2, changed state to up

Switch(config-if)#ip address 192.168.0.6 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#ex

Switch(config)#interface vlan 3

Switch(config-if)#

%LINK-5-CHANGED: Interface Vlan3, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan3, changed state to up

Switch(config-if)#ip address 192.168.0.7 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#ex

Switch(config)#interface vlan 4

Switch(config-if)#

%LINK-5-CHANGED: Interface Vlan4, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan4, changed state to up

Switch(config-if)#ip address 192.168.0.8 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#ex

Switch(config)#

Switch(config)#ex

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Switch#

Switch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#interface vlan 1

Switch(config-if)#ip address 192.168.0.5 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#

%LINK-5-CHANGED: Interface Vlan1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

Switch(config-if)#ex

Switch(config)#ex

Switch#ex

Switch>

Switch con0 is now available

Press RETURN to get started.

Switch>

# Command Prompt Result:

VLAN 1

VLAN 2

VLAN 4

VLAN 3